THE SOLAR SMART GARDEN

Complete build plans for a solar-powered, self-watering vegetable garden controlled by a Raspberry Pi

As designed and built at the Strathclyde Community Garden

      Winner, IChemE Young Engineers Award for Innovation and Sustainability (Food and Drink)

| What's inside |  |
| --- | --- |
| Full build guide | Every stage from empty beds to a garden that waters itself |
| Working software | Complete Python controller code, ready to copy onto your Pi |
| Parts list spreadsheet | Every component with typical UK prices and a tick-off column |
| Sizing calculators | Solar panel, battery and watering calculators in the spreadsheet |
| Troubleshooting + care | Symptom-by-symptom fixes and a season-by-season care plan |

| Skill level | Build time | Typical parts cost | Running cost |
| --- | --- | --- | --- |
| Confident beginner. No electronics experience assumed. | Two weekends | £265 to £360 depending on options | Nothing. The sun does the work. |

# Read me first (delete this page before you sell)

This is your master copy. Before exporting to PDF for customers, work through the yellow boxes. Each one marks a spot where your real build makes the guide stronger: your photos, your wiring diagram, your fertigation numbers, your exact part choices.

Yellow boxes to fill in: bed layout photo (Stage 1), wiring diagram and photos (Stage 2 and 3), your production code if it differs (Stage 4), your per-bed fertigation mass balances (Stage 5), bottle greenhouse photos (Stage 6), plus a photo of the finished garden for the cover.

Then: delete this page, update the award year if you want it on the cover, export as PDF, and upload the PDF plus the parts list spreadsheet as your Etsy digital files.

# Contents

1.  What you're building

2.  How it all works

3.  Safety first

4.  Parts and tools

5.  Stage 1: Water storage and delivery

6.  Stage 2: Solar power

7.  Stage 3: The brain

8.  Stage 4: The software

9.  Stage 5: Fertigation upgrade

10. Stage 6: Bottle micro-greenhouses

11. Commissioning checklist

12. Season-by-season care

13. Troubleshooting

14. Frequently asked questions

# 1. What you're building

By the end of this guide you'll have a vegetable garden that looks after itself. A small solar panel keeps a battery topped up. A Raspberry Pi checks the soil moisture in each bed and, when a bed gets thirsty, opens that bed's valve and runs a pump to drip water straight to the roots. It waters in the cool of the morning and evening, never floods, never forgets, and never sends you an electricity bill.

The design comes from a real, working installation: the community garden at the University of Strathclyde in Glasgow, where five raised beds run on exactly this system. The project won the IChemE Young Engineers Award for Innovation and Sustainability in the Food and Drink category, and this guide is the complete write-up of how to build your own.

## What the system does

- Waters up to five beds independently, each with its own moisture sensor and valve

- Runs entirely off-grid from one solar panel and a leisure battery

- Only waters when the soil actually needs it, inside time windows you choose

- Keeps a log of every watering so you can see what your garden has been up to

- Optionally doses liquid feed into the water (Stage 5), so beds get fed as well as watered

## What you need to know before starting

Honestly, not much. If you can use a screwdriver and follow a recipe, you can build this. All electrics run at 12 volts, the same as a car, so there's nothing here that can shock you. The software is provided in full; you copy it onto the Pi and change a few numbers. Where a stage involves a judgement call, this guide makes the call for you and tells you why.

# 2. How it all works

The system is four small subsystems that each do one job. Understand this page and the rest of the build is just assembly.

The flow of energy and water: sunlight hits the panel, the charge controller uses it to keep the battery charged, and the battery feeds two things: a 12 V rail for the pump and valves, and a small converter that makes clean 5 V for the Raspberry Pi. The Pi reads the soil sensors, decides which beds are thirsty, and flicks relays that switch the pump and valves on and off. Water flows from the storage tank, through a filter, through the pump, and out along drip lines to the beds.

| Subsystem | Its one job | Key parts |
| --- | --- | --- |
| Power | Turn sunlight into a steady 12 V supply, day and night | Solar panel, charge controller, battery, fuses |
| Water | Move water from the tank to the plant roots | Water butt, filter, 12 V pump, drip lines |
| Control | Decide when and where to water | Raspberry Pi, soil sensors, ADC boards, relay board |
| Plumbing logic | Send water to one chosen bed at a time | One 12 V solenoid valve per bed |

GOOD TO KNOW Why one bed at a time? A small pump can deliver strong, even dripping to one bed easily, but would struggle to feed five at once. Watering beds in sequence keeps the pump small, cheap and quiet, and the whole round still takes under fifteen minutes.

# 3. Safety first

Everything in this build runs at 12 V, which is classed as safe extra-low voltage. You can touch any bare wire in this system and feel nothing. The two things that deserve respect are the battery and the combination of water and electronics.

TAKE CARE Batteries store serious energy. A 12 V battery can push hundreds of amps into a short circuit. Drop a spanner across the terminals and it will weld itself in place and may start a fire. Fit the inline fuse on the battery's positive lead first, before connecting anything else, and keep terminals covered. Keep water and electronics apart. The Pi, ADC boards and relay board live in a sealed IP65 enclosure. Every cable enters through a gland, and every cable dips below the box before it enters (a drip loop), so rain runs off the cable instead of into the box. Charge indoors only with the right charger. If you ever top the battery up from the mains, use a proper 12 V leisure battery charger, in a dry place. Never connect mains electricity to any part of this system.

- Fuse both the battery lead and the load lead. Fuses are listed in the parts list.

- If you choose a lead-acid battery, house it upright in a ventilated box, not sealed in with the electronics.

- Mount the panel where wind can't lift it. Two brackets and four bolts beat cable ties every time.

- Route cables where a strimmer, a fork or a fox can't find them.

# 4. Parts and tools

The full shopping list, with typical UK prices and a tick-off column, lives in the parts list spreadsheet included with this guide. Here's the shape of it, and where you can save money.

| Subsystem | Main parts | Typical cost |
| --- | --- | --- |
| Power | 50 W panel, 10 A PWM charge controller, 12 V 20 Ah battery, fuses, bracket | £120 to £135 |
| Control | Raspberry Pi Zero 2 W, SD card, 12 V to 5 V converter, 2 ADC boards, 5 soil sensors, relay board, IP65 enclosure | £85 to £95 |
| Water | 12 V diaphragm pump, inline filter, 13 mm pipe, 4 mm micro-tube, 30 drippers, fittings, water butt | £55 to £80 (less if the butt is free) |
| Per-bed valves | Five 12 V solenoid valves | £35 to £45 |
| Sundries | Cable, glands, connectors, tape | £20 to £25 |

## Where to save money

- Single-zone version (saves about £50): skip the five valves and water all beds together from one line with one sensor in your thirstiest bed. You can add valves later without redoing anything.

- The water butt is often free. Councils and community groups regularly give them away, and food factories sell used IBC tanks cheaply.

- Don't save money on the sensors. Buy capacitive soil sensors, not the cheaper resistive type with exposed metal tracks. Resistive sensors corrode to uselessness in weeks.

## Tools

Drill with hole saw or step bit (for the enclosure and water butt), screwdrivers, wire strippers, crimping pliers or a small screwdriver for lever connectors, scissors or a pipe cutter for the drip line, adjustable spanner, and a multimeter. A £10 multimeter is fine and will pay for itself in the first hour of fault-finding.

# 5. Stage 1: Water storage and delivery

Start with water because it's the most satisfying stage: by the end of it you can water the whole garden by touching two wires together.

## The tank

A standard 210 litre water butt is plenty. Raise it on blocks or a stand so the outlet tap sits above the pump inlet; the pump will do the real work, but starting flooded means it never struggles to prime. Fit the inline filter between the tap and the pump. This is not optional. Drippers have tiny outlets and one flake of algae will block one for good.

## How much water do you actually need?

Work it out once and the pump and tank sizes fall out automatically. With 2 litre-per-hour drippers and six drippers per bed:

| Question | Sum | Answer |
| --- | --- | --- |
| Flow when one bed waters | 6 drippers × 2 L/h | 12 L/h |
| Water per bed per day (summer) | 12 L/h × 15 minutes | 3 litres |
| Whole garden per day | 5 beds × 3 L | 15 litres |
| How long a full butt lasts | 210 L ÷ 15 L per day | About 2 weeks |

The watering calculator in the spreadsheet does these sums for your own bed count and dripper choice.

## The pump

A 12 V diaphragm pump rated around 4 litres per minute is ideal. Diaphragm pumps self-prime, tolerate the odd dry moment, and produce enough pressure (2 to 3 bar) to push water uphill and through long runs. They're the same pumps used in campervans, so they're cheap and everywhere.

## The pipework

- Run 13 mm LDPE supply pipe from the pump output along the front of the beds. This is the motorway.

- At each bed, tee off through that bed's solenoid valve (or straight through, for the single-zone version).

- After each valve, run 4 mm micro-tube into the bed and punch in six drippers per bed, one beside each plant or evenly spaced.

- Flush the whole system with the ends open before fitting the last end plugs, to clear any swarf or grit.

GOOD TO KNOW Test the pump now. Touch its wires to the battery (through the fuse) and watch the beds drip. Every dripper should tick along steadily. If the far bed is weak, your drippers aren't pressure-compensating; either buy the compensating type or shorten the run.

SLOT IN YOUR OWN CONTENT HERE Add a labelled photo or sketch of your own bed layout here: tank position, pipe route, valve positions, dripper spacing. One good overhead photo with arrows drawn on it is worth a page of text.

# 6. Stage 2: Solar power

Sizing solar looks like black magic but it's three sums: how much energy you use a day, how big a panel you need to harvest it, and how big a battery you need to ride out cloudy spells.

## Step 1: Your daily energy budget

| Load | Power | Hours per day | Energy per day |
| --- | --- | --- | --- |
| Raspberry Pi Zero 2 W (always on) | 1.2 W | 24 | 29 Wh |
| Pump (36 W while running) | 36 W | 0.25 | 9 Wh |
| Valves (one at a time while watering) | 6 W | 0.25 | 1.5 Wh |
| Charge controller's own use | 0.12 W | 24 | 3 Wh |
| Total |  |  | About 42 Wh per day |

## Step 2: The panel

In the UK growing season (March to October) you can bank on roughly 3 peak-sun hours per day on average. Allowing 70 percent system efficiency for cloud, cable losses and battery charging:

Panel needed = 42 Wh ÷ 3 h ÷ 0.7 ≈ 20 W. Buy a 50 W panel. The extra headroom means the battery recovers fast after a grey week and the whole system shrugs off a Scottish April.

GOOD TO KNOW Want it running through winter too? UK December sunshine drops to under 1 peak-sun hour per day, so you'd want a 100 W panel and a bigger battery. Most gardeners simply shut the system down from November to February (Section 12), because nothing needs watering then anyway.

## Step 3: The battery

Three days of autonomy covers a properly dreich spell: 42 Wh × 3 = 126 Wh. A lead-acid leisure battery shouldn't be run below half charge, so you want at least 250 Wh of capacity, which at 12 V is about 20 Ah. A 12 V 20 Ah AGM battery is the budget pick; a 12 V 12 Ah lithium (LiFePO4) battery costs a little more, weighs half as much and can use 80 percent of its capacity.

## Step 4: The charge controller and wiring order

A 10 A PWM charge controller is perfect at this scale (MPPT controllers only earn their extra cost above about 100 W of panel). The wiring order matters and is the single most common first-day mistake:

- Battery to controller first. Through the 10 A inline fuse on the positive lead. The controller wakes up and learns the battery voltage.

- Panel to controller second. Cover the panel with cardboard while you connect it so it isn't live in your hands.

- Loads to the controller's load output last. The 12 V feed to the relay board and the 5 V converter comes from the controller's load terminals, not straight off the battery, so the controller can protect the battery from over-discharge.

Use 1.5 mm² cable for everything on the 12 V side and keep runs under 3 metres where you can. Short and fat beats long and thin.

SLOT IN YOUR OWN CONTENT HERE Drop in your wiring diagram here, plus a photo of your controller with the three connections labelled. If you drew your diagram for the DesignSpark article, reuse it. This is the page customers will keep coming back to, so make it the clearest one in the guide.

# 7. Stage 3: The brain

Time to give the garden a nervous system. Everything in this stage lives inside one sealed enclosure, mounted on a post or fence in light shade.

## The computer

A Raspberry Pi Zero 2 W is the sweet spot: it sips power (about 1.2 W), has built-in Wi-Fi for setup and log collection, and costs under £20. A Pi 3A+ or Pi 4 also works if you have one in a drawer, at the cost of a bigger power budget. Power the Pi from a 12 V to 5 V buck converter (3 A, USB output) fed from the controller's load terminals. Never try to power a Pi from the panel directly.

## Reading the soil

Capacitive soil moisture sensors output a voltage, and the Pi has no analogue inputs, so an ADS1115 analogue-to-digital board sits in between and talks to the Pi over two wires (the I2C bus). One ADS1115 reads four sensors; for five beds you fit two boards and solder the ADDR jumper on the second one so they have different addresses (0x48 and 0x49). Bury each sensor to its line in the middle of its bed, and seal the top of the sensor and its cable joint with self-amalgamating tape or hot glue, because the boards themselves hate rain.

## Switching the power

An 8-channel opto-isolated 5 V relay board switches the pump and the five valves (six channels used, two spare for whatever you dream up later). The Pi's GPIO pins drive the relay inputs directly, and the relays' switched side carries the 12 V to each device.

## Suggested wiring map

| Connection | From | To |
| --- | --- | --- |
| Pump power | Relay channel 1 | Pump positive lead |
| Bed 1 to 5 valves | Relay channels 2 to 6 | Each valve's positive lead |
| Pump relay signal | Pi GPIO 17 | Relay IN1 |
| Valve relay signals | Pi GPIO 5, 6, 13, 19, 26 | Relay IN2 to IN6 |
| Sensor data | ADS1115 SDA and SCL | Pi GPIO 2 (SDA) and GPIO 3 (SCL) |
| Sensors 1 to 4 | ADS1115 board 1, inputs A0 to A3 | Sensor signal wires |
| Sensor 5 | ADS1115 board 2, input A0 | Sensor signal wire |

GOOD TO KNOW Label everything as you go. Masking tape and a marker on both ends of every wire turns future fault-finding from an afternoon into a minute.

SLOT IN YOUR OWN CONTENT HERE Photo slot: the inside of your enclosure, tidy and labelled, plus one photo of a sensor installed in a bed. Customers trust guides where they can see the real thing.

# 8. Stage 4: The software

The complete controller program is below and is also worth typing into a file called garden.py as you read it. It checks every bed twice a day inside your chosen time windows, waters any bed below its moisture target, stops the moment the soil is wet enough or a safety timer runs out, and logs everything to a CSV file you can open in any spreadsheet.

## Set up the Pi (fifteen minutes)

- Flash Raspberry Pi OS Lite to the SD card with Raspberry Pi Imager, using its settings gear to set a username (pi), your Wi-Fi and enable SSH.

- Boot the Pi, connect over SSH, and enable the I2C bus: sudo raspi-config, then Interface Options, then I2C, then Yes.

- Install the sensor library: sudo apt update && sudo apt install -y python3-pip, then pip3 install adafruit-circuitpython-ads1x15.

- Check the Pi can see both ADC boards: sudo apt install -y i2c-tools, then i2cdetect -y 1 should show 48 and 49.

## The controller: garden.py

#!/usr/bin/env python3 """Solar Smart Garden - irrigation controller. Reads capacitive soil moisture sensors through two ADS1115 boards and waters each bed through its own solenoid valve when the soil is drier than its target. One bed waters at a time. """ import csv import time from datetime import datetime import board import busio from adafruit_ads1x15.ads1115 import ADS1115, P0, P1, P2, P3 from adafruit_ads1x15.analog_in import AnalogIn from gpiozero import OutputDevice # ---------- settings you will edit ---------- PUMP_GPIO = 17 BEDS = { "Bed 1": {"adc": 0, "ch": P0, "valve_gpio": 5, "dry": 21500, "wet": 9500, "target": 45, "max_seconds": 120}, "Bed 2": {"adc": 0, "ch": P1, "valve_gpio": 6, "dry": 21500, "wet": 9500, "target": 45, "max_seconds": 120}, "Bed 3": {"adc": 0, "ch": P2, "valve_gpio": 13, "dry": 21500, "wet": 9500, "target": 45, "max_seconds": 120}, "Bed 4": {"adc": 0, "ch": P3, "valve_gpio": 19, "dry": 21500, "wet": 9500, "target": 45, "max_seconds": 120}, "Bed 5": {"adc": 1, "ch": P0, "valve_gpio": 26, "dry": 21500, "wet": 9500, "target": 45, "max_seconds": 120}, } WATER_WINDOWS = [(6, 8), (19, 21)] # water 06:00-08:00 and 19:00-21:00 only CHECK_EVERY_MINUTES = 30 LOG_FILE = "/home/pi/garden_log.csv" # --------------------------------------------- i2c = busio.I2C(board.SCL, board.SDA) adcs = [ADS1115(i2c, address=0x48), ADS1115(i2c, address=0x49)] # Most relay boards switch ON when the input is pulled LOW. # If a relay clicks ON the moment the Pi boots, change # active_high=False to active_high=True below. pump = OutputDevice(PUMP_GPIO, active_high=False, initial_value=False) valves = {name: OutputDevice(cfg["valve_gpio"], active_high=False, initial_value=False) for name, cfg in BEDS.items()} def moisture_percent(cfg): """Average ten readings and map raw counts to 0-100 %.""" chan = AnalogIn(adcs[cfg["adc"]], cfg["ch"]) reading = sum(chan.value for _ in range(10)) / 10 pct = (cfg["dry"] - reading) / (cfg["dry"] - cfg["wet"]) * 100 return max(0.0, min(100.0, pct)) def in_water_window(): hour = datetime.now().hour return any(start <= hour < end for start, end in WATER_WINDOWS) def log(bed, moisture, watered_s): with open(LOG_FILE, "a", newline="") as f: csv.writer(f).writerow([datetime.now().isoformat(timespec="seconds"), bed, round(moisture, 1), watered_s]) def water(bed, cfg): """Open this bed's valve, run the pump, stop when wet or timed out.""" valves[bed].on() pump.on() start = time.time() try: while time.time() - start < cfg["max_seconds"]: time.sleep(5) if moisture_percent(cfg) >= cfg["target"]: break finally: pump.off() valves[bed].off() return round(time.time() - start) def main(): while True: if in_water_window(): for bed, cfg in BEDS.items(): pct = moisture_percent(cfg) seconds = water(bed, cfg) if pct < cfg["target"] else 0 log(bed, pct, seconds) time.sleep(2) time.sleep(CHECK_EVERY_MINUTES * 60) if __name__ == "__main__": main()

## Calibrate your sensors (do this once)

Every sensor reads slightly differently, so teach the program what dry and wet mean for each one. With a sensor connected, read its raw value in dry air, then with its blade in a glass of water. Put those two numbers into that bed's dry and wet settings at the top of garden.py. A target of 45 percent suits most vegetables; thirsty crops like courgettes like 55, Mediterranean herbs prefer 30.

## Make it run at boot

Save the following as /etc/systemd/system/garden.service, then run sudo systemctl enable --now garden. From that moment the garden waters itself, survives power cuts, and restarts on any error.

[Unit] Description=Solar Smart Garden irrigation controller After=network.target [Service] ExecStart=/usr/bin/python3 /home/pi/garden.py Restart=always RestartSec=10 User=pi [Install] WantedBy=multi-user.target

SLOT IN YOUR OWN CONTENT HERE If your production system runs different code (a dashboard, cloud logging, per-bed fertigation control), either swap it in here or add it as a bonus appendix labelled 'the advanced version'. Selling both versions in one guide is a genuine selling point.

# 9. Stage 5: Fertigation upgrade

Fertigation means feeding through the irrigation water, and it's what separates this system from every timer-and-hose kit on the market. Because each bed has its own valve, each bed can receive its own feed schedule: hungry brassicas get more, herbs get almost none.

## The simple route: a venturi injector

A venturi injector is a £10 fitting that tees into the main line and sucks concentrated liquid feed from a small container whenever water flows past. Fit it after the pump, dial the suction valve to a weak mix, and every bed gets gently fed at every watering. No electronics, no moving parts.

## The precise route: per-bed dosing

Add a small 12 V peristaltic dosing pump on a spare relay channel, drawing from a nutrient reservoir into the main line. Because the software waters one bed at a time, running the dosing pump only during chosen beds' watering slots gives true per-bed feeding. Two lines of code per bed in the water() function is all it takes.

## Getting the dose right

The sum is a mass balance: target concentration times litres delivered equals grams of feed needed. Say a bed gets 3 litres per day and you want 150 ppm of nitrogen, which is 150 milligrams per litre. That's 450 milligrams of nitrogen per day. If your feed is 6 percent nitrogen, the bed needs 7.5 grams of feed solution per day, and you set the dosing time to deliver exactly that.

SLOT IN YOUR OWN CONTENT HERE This is where your award-winning per-bed mass balance tables go. Present them exactly as you built them: one table per bed with crop, daily water volume, target nutrient concentrations and the resulting dose. This section is the single biggest differentiator of the whole product, so give it your best material.

# 10. Stage 6: Bottle micro-greenhouses

A free upgrade that came straight from the Strathclyde garden: cut the bottom off a clear 2 or 5 litre bottle and push it into the soil over a seedling, keeping the cap loose or off for ventilation. Each plant gets its own greenhouse, a few degrees warmer and safe from slugs, made from something rescued from the recycling. In Glasgow this trick keeps young citrus and tender seedlings alive weeks earlier in spring. Push the bottle rim a couple of centimetres into the soil so wind can't lift it, drip line running underneath as normal, and remove it once the plant fills the space.

SLOT IN YOUR OWN CONTENT HERE One photo of a bottle greenhouse in place, ideally with a healthy plant inside. Buyers love this section because it costs nothing.

# 11. Commissioning checklist

Do these in order on switch-on day and nothing will surprise you.

- Fuse fitted on the battery positive lead. Terminals covered.

- Battery connected to controller. Controller display or LED wakes up.

- Panel connected (covered with cardboard, then uncovered). Charging light comes on in daylight.

- 5 V converter connected to load terminals. Pi boots; you can SSH in.

- i2cdetect shows 48 and 49.

- Run garden.py by hand with the pump hose aimed into a bucket. Relays click in sequence, one bed at a time.

- Connect the real pipework. Watch a full watering round. Check every dripper drips and every joint is dry.

- Enable the systemd service. Pull the Pi's power for ten seconds, restore it, and confirm the system comes back by itself.

- Leave it alone for 48 hours, then read garden_log.csv. If the log shows sensible moisture numbers and short, regular waterings, you're done. It works. Go and do something else with your evenings.

# 12. Season-by-season care

| Season | What to do |
| --- | --- |
| Spring | Wipe the panel, reconnect the battery, flush the lines with the end plugs off, recalibrate any sensor that over-wintered in soil, restart the service. |
| Summer | Skim the filter monthly, top up the butt in dry spells, nudge moisture targets up for fruiting crops. |
| Autumn | Drop watering to once a day, then shut down after the last harvest: drain the pump and lines so frost can't split them. |
| Winter | Bring the battery somewhere frost-free and give it a top-up charge monthly (skip the monthly charge if it's lithium). Leave the panel connected to the controller or store it inside. The Pi is happy in the sealed box. |

# 13. Troubleshooting

| Symptom | Most likely cause | Fix |
| --- | --- | --- |
| Nothing powers up | Blown fuse or controller in low-voltage cutoff | Check the fuse, then check battery voltage. Below 11.8 V, let the panel charge it for a day. |
| Pi runs but sensors read 0 or garbage | I2C not enabled, or SDA and SCL swapped | Re-run raspi-config, then i2cdetect. Swap the two wires if 48 and 49 don't appear. |
| One bed never waters | Sensor calibration, stuck valve, or kinked micro-tube | Read that bed's raw sensor value; re-do dry and wet numbers. Listen for the valve click. Trace the tube. |
| One bed waters constantly | Sensor pulled out of the soil or cable damaged | A sensor in air reads bone dry. Re-seat it and check the cable. |
| Relays click but pump silent | 12 V not reaching the relay's switched side | Check the wire from the controller load terminal to the relay common contacts. |
| Weak dripping at the far bed | Pressure drop along the run | Fit pressure-compensating drippers or move the pump nearer the middle of the run. |
| Drippers blocked | Filter skipped or dirty | Fit or clean the inline filter, then soak drippers in warm water and vinegar. |
| Battery flat every morning | Panel shaded, undersized, or battery worn out | Check nothing shades the panel between 10am and 2pm. Measure battery voltage at dusk and dawn; a drop below 12 V overnight with these small loads means the battery is failing. |
| Relay board clicks everything ON at boot | Relay board is active-low (most are) and code set active-high, or vice versa | Flip the active_high setting in garden.py as noted in the code comments. |
| Water in the enclosure | Missing drip loops or an unsealed gland | Dry everything thoroughly, re-make the cable entries, add drip loops, and drop a desiccant sachet in the box. |

# 14. Frequently asked questions

Can I run it from a tap instead of a tank? Yes. Drop the pump, keep the valves, and fit a pressure reducer, because mains pressure will pop 4 mm fittings. The tank version is nicer for water metering and rain harvesting.

I only have two beds, or I have eight. The design scales either way. Fewer beds just means spare relay channels; more than eight means a second relay board and a bigger pump, and the spreadsheet calculators re-size the solar for you.

Does it work on a balcony? Beautifully. One zone, a 25 litre container instead of a butt, a 20 W panel, and pots instead of beds.

What does it cost to run? Nothing. There is no plug. Your only recurring cost is the water in the butt, and if that's rainwater, the whole thing runs on weather.

My garden gets no direct sun. Mount the panel remotely: on a shed roof, a fence, a pole. Panel to controller cable can run 5 metres or more if you step up to 2.5 mm² cable.

Do I need to know Python? No. You change numbers at the top of one file. Everything you'd want to tweak (watering windows, moisture targets, safety timeouts) is a labelled setting.

Happy growing. If you build one, the plants won't thank you, but they'll show it.